SECTION	114	SAFETY PROCEDURES

STANDARD OPERATING POLICY

POLICY 	114.64

FIRE DEPARTMENT

##### BACKING SAFETY

PURPOSE

The purpose of this guideline is to outline proper safety practices to be used when backing Fire Department vehicles and maneuvering in close-quarters situations.

POLICY

The policy of the McKinney Fire Department concerning the backing and close-quarter maneuvering of Department vehicles will be as follows:

Fire department vehicles will be backed and maneuvered without hitting a person, object, or other vehicle.

Using the information provided in this procedure virtually eliminates the possibility of hitting a person, object, or other vehicle while backing or maneuvering in close-quarters.  Therefore, all Department personnel will be expected to follow the guidelines contained in Sections III and IV of this procedure and, in addition, take all reasonable and necessary actions to comply with this policy.  Hitting a person, object or other vehicle while backing a department vehicle is an unacceptable event.

This policy will apply when backing a department vehicle, when maneuvering Fire Department apparatus and ambulances with restricted rear visibility, or in situations where vehicle clearances are uncertain.

Accountability

The company officer or acting officer will be responsible for all vehicles assigned to his/her company and the overall compliance of his/her company personnel with the guidelines provided in this procedure at all times.  Each vehicle driver will be responsible for the safe operation of his or her vehicle at all times.  This includes ensuring the vehicle is safely backed and maneuvered without incident.

Procedure

The following guidelines will be used to ensure that Department vehicles are backed and maneuvered safely and without incident:

A.	The need to back a department vehicle will be avoided when at all possible by positioning the vehicle so to have a forward egress route.  Attempt to avoid situations that require vehicle backing or close-quarter maneuvering such as dead ends, cul-de-sacs, and routine parking situations whenever possible.

B.	When backing is unavoidable, a minimum of one ground guide person will be assigned and used.  It is the responsibility of the officer, if present, to assure that a ground guide is assigned when necessary.  In addition, ground guide(s) will be used when a vehicle must negotiate forward turns with restrictive side or rear clearances and where clearances above or beneath the vehicle are uncertain.

C.	The vehicle driver will determine the necessary number of ground guide personnel, with a minimum of one(1), to ensure the vehicle is backed and maneuvered safely without striking a person, object, or other vehicle.

D.	It is the driver’s responsibility to verbally communicate their planned vehicle backing route and procedures with the ground guide(s) prior to backing.

E.	It is advisable that the ground guide has a portable radio with them when spotting a vehicle.  When the driver and backer agree to use a radio channel as the primary means of communicating during backing or maneuvering, it is the ground guide’s responsibility to initially contact the driver by radio and receive an acknowledgment from the driver to insure that radio communication is established.  The vehicle shall not be backed until this two-way radio communication has been verified.

F.	When a ground guide is assigned, the vehicle will not be backed until the ground guide(s) is in position and have signaled visually or by radio, their approval to start backing.

G.	Due to the potential for the existence of a significant blind spot on the passenger’s side of a backing vehicle, consideration should be given to positioning a ground guide on the right side of the backing vehicle, within view of the driver when looking in their right side mirror. Under these situations, the driver can use their left mirror to view the left side of their vehicle while the ground guide monitors the right side.

H.	The vehicle will be immediately stopped any time one of the following situations exists:

1.	The driver loses sight of or breaks visual contact with the ground guide.

- The driver is confused or unsure of the signals being provided by the ground guide.

- The driver receives a visual, verbal, or radio communication to STOP.

The driver may continue backing the vehicle only after the above situations are corrected and the communication to continue backing is given by the ground guide(s).

I.	A highway safety vest or turnout coat shall be worn by all ground guides during the backing process.  When backing a vehicle in or near moving traffic, the vehicle's emergency lights can be operated at the discretion of the driver or officer unless their operation will in any way hinder the vision of the ground guide.

J.	When backing vehicles at night, ground guides should consider turning on and properly aiming the vehicle's rear spotlights or scene lights (if equipped with such lights) to better illuminate the backing area.  In addition, ground guides may use flashlights to assist the driver, but at no time shall the flashlight beam be directed toward the vehicle mirrors.

K.	Ground guides are not permitted to ride on the backing vehicle while in motion.

L.	In circumstances where the vehicle is occupied by only the driver and backing without a guide is unavoidable, the driver will accomplish three basic tasks; 1) assure that a complete 360-degree survey of the area around the vehicle has been conducted to determine what, if any, obstructions are present. 2) attempt to use any available Department personnel, police officer, City employee, or other individuals to serve as improvised ground guides, 3) where no personnel are available to assist, the vehicle may be cautiously backed while watching for pedestrians, vehicles, and obstructions that may enter the path of the moving vehicle. It is the driver’s responsibility to assure that their audible back-up alarm is sounding before moving the vehicle.

M.  	Backing of a vehicle while responding to or operating at emergency incidents necessitates consideration for safety at all times and a common sense approach to vehicle backing.  Every reasonable and prudent effort shall be made to comply with these backing or close quarter maneuvering safety guidelines during emergency response or incident scene conditions.

Signals

The following signals are recommended to use by all Department personnel when serving as ground guides:

Straight Back:  One hand only above the head with palm toward face, waving back. Hold arm above your eye level and outside of your body’s outline to improve visibility of the signaling.

Turn: Both arms pointing the same direction with index fingers extended and pointing arm outside of the body’s outline to improve visibility of the signaling.

The vehicle will be backed in the direction of pointing.  See Figure 2.

Stop:  Both arms above the head with hands clinched together or arms folded into the shape of an ‘X’.  See Figure 3.

Verbal:  Ground guides may use radio communications to supplement hand signals at any time.